Parul Institute of Engineering & Technology

# Electronics & Communication Engineering Department

ASSIGNMENT-6

# Subject Name: DIGITAL ELECTRONICS

Subject Code: 203105201

Programmable Logic Devices:

- Introduction to PLD and its types.

- Compare RAM, ROM and EPROM.

- Compare PLA and PAL.

- Write short note on Programmable Logic Arrays.

- Write short note on State machine on a Chip.

- What is FPGA? Explain its basic structure.

- A combinational circuit is defined by functions:

F1 (A, B, C) = Σ (3, 5, 6, 7)

F2 (A, B, C) = Σ (0, 2, 4, 7)

Implement the circuit with PLA having three inputs, four product term and two outputs.

- Using the simplified connection format of PLA, Show how an 8×1 PROM should be programmed to implement the logic function

F (A, B, C) = Σ (1, 4, 5, 7)

- Show how the PLA would be programmed to implement the functions. PLA having three inputs, four product term and two outputs.

F1=A(BC)’+ABC F2=ABC+(AB)’+AC’

- Using the simplified connection format of PLA, Show how an 8×1 PROM should be programmed to serve as a look-up table for the odd parity-bit of a 3 bit number? Use the simplified connection format of PLA.

- Show how the FPLA Circuit would be programmed to implement F1= A’BC’+B’C+ABC

- Implement the state diagram shown in fig. using PLS (Programmable Logic Sequencer )

0/0

1/0

0/1

00

10

0

0/1

0/1

1/0

1/0

01

11

1/